PID

Understanding the PID (Proportional-Integral-Derivative) control components can be simplified by breaking down each component and relating it to a practical example:

- Proportional (P):

  - Function: Reacts to the current error.

  - Explanation: The proportional term produces an output value that is proportional to the current error value. The proportional response can be adjusted by multiplying the error by a constant known as the proportional gain (KpK_pKp​).

  - Example: Imagine you are driving a car and need to stay in the center of a lane. If you are 2 feet to the left of the center, you might turn the wheel slightly to the right to correct your position. The more off-center you are, the more you turn the wheel.

- Integral (I):

  - Function: Reacts to the sum of past errors.

  - Explanation: The integral term is concerned with the accumulation of past errors. It integrates the error over time, adding up the past errors and multiplying by a constant known as the integral gain (KiK_iKi​). This helps eliminate residual steady-state error that a purely proportional controller would leave.

  - Example: Continuing with the driving example, if you have been off-center for a while, even if only slightly, you might start turning the wheel a bit more to correct for the accumulated error over time.

- Derivative (D):

  - Function: Reacts to the rate of change of the error.

  - Explanation: The derivative term produces an output value that is proportional to the rate of change of the error. It predicts the future trend of the error based on its current rate of change and multiplies this by a constant known as the derivative gain (KdK_dKd​). This helps dampen the system and reduce overshoot.

  - Example: In the driving scenario, if you notice you are starting to drift off-center more quickly, you would preemptively adjust the wheel to counteract this increasing error.

### Practical Visualization:

- Proportional (P): Think of it as making corrections based on how far you are from the target. If you're far off, you make a big correction.

- Integral (I): Think of it as making corrections based on how long you have been away from the target. If you've been off-center for a long time, even small corrections add up.

- Derivative (D): Think of it as making corrections based on how quickly you're moving away from the target. If you start drifting quickly, you correct more aggressively to prevent overshooting.

Let's try to simplify the concepts further by using a relatable analogy: controlling the temperature of a shower.

### Proportional (P):

- Analogy: Imagine you are adjusting the shower temperature. When the water is too cold, you turn the knob toward hot. The colder the water, the more you turn the knob.

- Key Point: The adjustment (knob turn) is directly proportional to the error (difference between desired and actual temperature).

- Impact: If the water is 5 degrees too cold, you make a small adjustment. If it's 10 degrees too cold, you make a larger adjustment.

### Integral (I):

- Analogy: Now, imagine you notice the water is consistently slightly too cold over several minutes. Even if the error is small at any given moment, the fact that it has been consistently cold means you should gradually increase the temperature more.

- Key Point: The adjustment is based on the accumulation of past errors. If the water has been too cold for a while, you turn the knob more to compensate.

- Impact: If the water has been 1 degree too cold for 10 minutes, you make a significant adjustment to bring it up to the desired temperature.

### Derivative (D):

- Analogy: Now, imagine you suddenly feel the water getting colder quickly. To prevent it from becoming too cold, you quickly adjust the knob toward hot.

- Key Point: The adjustment is based on the rate of change of the error. If the temperature is changing quickly, you react more aggressively.

- Impact: If the water temperature drops by 2 degrees in 2 seconds, you make a quick adjustment to counteract the rapid change.

### Combining Them:

- Proportional (P): Corrects based on current error.

  - If water is too cold by 5 degrees, turn the knob proportionally.

- Integral (I): Corrects based on accumulated past errors.

  - If water has been too cold by 1 degree for a long time, make a larger adjustment.

- Derivative (D): Corrects based on the rate of change of the error.

  - If water temperature is dropping quickly, make a rapid adjustment to prevent further cooling.

### Formula:

Control Output=Kp×Error+Ki×Sum of Past Errors+Kd×Rate of Change of Error\text{Control Output} = K_p \times \text{Error} + K_i \times \text{Sum of Past Errors} + K_d \times \text{Rate of Change of Error}Control Output=Kp​×Error+Ki​×Sum of Past Errors+Kd​×Rate of Change of Error

### Practical Visualization:

- Proportional (P): Imagine making immediate adjustments based on how far off the temperature is from the desired setting.

- Integral (I): Think of making gradual adjustments based on how long the temperature has been off.

- Derivative (D): Consider making quick adjustments to counteract rapid changes in temperature.

### Example with Numbers:

- Proportional (P):

  - Desired temperature: 40°C

  - Current temperature: 35°C

  - Error: 5°C

  - Adjustment: Kp×5K_p \times 5Kp​×5

- Integral (I):

  - Desired temperature: 40°C

  - Error over time: consistently 1°C off for 10 minutes

  - Adjustment: Ki×(1°C * 10 minutes)K_i \times \text{(1°C * 10 minutes)}Ki​×(1°C * 10 minutes)

- Derivative (D):

  - Desired temperature: 40°C

  - Error change: 2°C drop in 2 seconds

  - Adjustment: Kd×(2°C / 2 seconds)K_d \times \text{(2°C / 2 seconds)}Kd​×(2°C / 2 seconds)

### Summary:

- P: Corrects based on the current temperature difference.

- I: Corrects based on the history of the temperature difference.

- D: Corrects based on the speed at which the temperature difference is changing.

Using this analogy, you can think of tuning the PID controller like you would adjust a shower to maintain a comfortable temperature: responding to immediate discrepancies (P), past trends (I), and rapid changes (D).

4o

##### You said:

Let's use a simpler analogy to understand each component of a PID controller. Imagine you are riding a bicycle and trying to stay on a straight path. The goal is to keep the bicycle in the middle of the path, and the handlebars are what you use to make adjustments. The error is how far off you are from the center.

### Proportional (P):

- Concept: How far off are you from the center?

- Analogy: If you are 5 inches to the left of the center, you turn the handlebars to the right. The further you are from the center, the more you turn.

- Key Idea: The adjustment is directly proportional to the current error. If you're far from the center, make a big correction. If you're close, make a small correction.

### Integral (I):

- Concept: How long have you been off-center?

- Analogy: If you’ve been 1 inch off to the left for a long time, it means you need to make a correction. This term sums up all the past errors to determine the adjustment.

- Key Idea: The adjustment is based on the accumulation of past errors. If you've been off-center for a long time, even slightly, you make a larger correction to bring yourself back to the center.

### Derivative (D):

- Concept: How quickly are you moving off-center?

- Analogy: If you suddenly start drifting to the left quickly, you turn the handlebars to the right to counteract this movement.

- Key Idea: The adjustment is based on the rate of change of the error. If you are drifting quickly, you make a bigger correction to prevent overshooting.

### Bringing It All Together:

- P (Proportional): Imagine you're always looking at your current position relative to the center. If you see you're off-center, you turn the handlebars proportionally to how far off you are.

  - Example: If you're 10 inches off, you turn the handlebars a lot. If you're 2 inches off, you turn them a little.

- I (Integral): Now, imagine you also keep track of how long you've been off-center. If you've been off-center for a while, even just a little bit, you start making a correction based on this accumulated error.

  - Example: If you've been slightly off-center for a long time, you make a steady correction to bring yourself back.

- D (Derivative): Finally, imagine you also pay attention to how quickly you're moving away from the center. If you notice you're starting to drift quickly, you make a preemptive correction to stop this drift.

  - Example: If you start drifting left quickly, you turn right quickly to stop the drift.

### Summary in One Sentence:

- P (Proportional): Corrects based on how far you are from the target.

- I (Integral): Corrects based on how long you have been off target.

- D (Derivative): Corrects based on how quickly you are moving away from the target.

### Another Visual Analogy:

- Proportional (P):

  - Imagine a spring attached to the centerline of the path. The more you move away from the center, the stronger the spring pulls you back.

- Integral (I):

  - Imagine a rubber band that stretches more the longer you are away from the center. Over time, it pulls you back more strongly.

- Derivative (D):

  - Imagine a damper (like shock absorbers in a car) that resists quick movements. If you start moving away quickly, it slows you down to prevent overshooting.

### Mathematical Formula:

Output=Kp×Error+Ki×∑Error+Kd×d(Error)dt\text{Output} = K_p \times \text{Error} + K_i \times \sum \text{Error} + K_d \times \frac{d(\text{Error})}{dt}Output=Kp​×Error+Ki​×∑Error+Kd​×dtd(Error)​

- KpK_pKp​: Proportional gain - adjusts based on current error.

- KiK_iKi​: Integral gain - adjusts based on accumulated error.

- KdK_dKd​: Derivative gain - adjusts based on the rate of change of error.

This formula combines the three corrections to give you a balanced response that keeps you on target by considering the present, past, and future trends of the error.

4o

##### You said:

canu use teh hstorical context of theri intordcution to contoel systems becuse i stoll dont get ti

###### ChatGPT said:

ChatGPT

Sure! Let's delve into the historical context and practical applications of PID controllers to give you a clearer understanding of how they work and why they were developed.

### Historical Context of PID Controllers

- Early Industrial Control (1910s-1930s):

  - Proportional Control: Early control systems, such as those used in steam engines and boilers, used proportional control. Engineers noticed that simply adjusting the control based on the current error (how far off the desired value) was beneficial. However, proportional control alone couldn't always maintain the desired setpoint precisely, especially in systems with varying loads or disturbances.

- The Need for More Precision (1930s-1940s):

  - Integral Control: To address the limitations of proportional control, the integral component was introduced. Integral control considers the accumulation of past errors. This means it adjusts the control action to eliminate any steady-state error that might persist when using proportional control alone.

  - Example: In temperature control for a boiler, if the temperature consistently stays a bit below the setpoint, the integral action will gradually increase the heat input to bring the temperature to the desired setpoint.

- Response to Rapid Changes (1940s-1950s):

  - Derivative Control: As control systems became more complex, the derivative component was added. Derivative control predicts the future trend of the error by considering its rate of change. This helps to dampen the response, reducing overshoot and oscillations.

  - Example: In an automatic steering system for a ship, if the ship starts to turn too quickly due to a sudden disturbance, the derivative action will apply a counteracting force to smooth out the turn.

### Practical Application: PID in Automatic Steering Systems

Let's consider the development of automatic steering systems for ships, which were some of the earliest applications of PID control.

- Proportional Control:

  - Scenario: Imagine a ship needs to maintain a straight course. If the ship deviates from its course, the rudder is turned to correct the error.

  - Action: If the ship is 10 degrees off course, the rudder is turned proportionally to correct it. The further off course, the more the rudder turns. However, if there's a constant disturbance (like a steady wind), proportional control might not be enough to keep the ship on course.

- Integral Control:

  - Scenario: Over time, the ship might drift off course due to the persistent wind. The integral action sums up these errors over time.

  - Action: The rudder adjustment increases over time to correct for the accumulated error, ensuring the ship eventually returns to and stays on the desired course.

- Derivative Control:

  - Scenario: If the ship starts to turn too quickly due to a sudden wave, the derivative action comes into play.

  - Action: The rudder is adjusted to counteract the rapid change, preventing the ship from overshooting the desired course and oscillating back and forth.

### Simplified Analogy with Historical Context

- Proportional (P):

  - Historical Use: Early feedback systems in steam engines.

  - Analogy: Adjusting the valve based on how far the engine's speed is from the desired speed.

  - Problem: Alone, it can't eliminate steady-state errors (e.g., the engine might run slightly faster or slower than desired).

- Integral (I):

  - Historical Use: Added to improve long-term accuracy.

  - Analogy: Continuously adjusting the valve to correct any ongoing difference between the engine's speed and the desired speed.

  - Problem: Can lead to overshoot and oscillation if not balanced properly.

- Derivative (D):

  - Historical Use: Helps to smooth out rapid changes and improve stability.

  - Analogy: Adjusting the valve to counteract rapid changes in the engine's speed.

  - Benefit: Prevents the engine from oscillating or overshooting the desired speed.

### Summary

- Proportional (P): Corrects the error based on the current deviation.

  - Example: Turn the rudder more if the ship is further off course.

- Integral (I): Corrects based on the accumulation of past errors.

  - Example: Adjust the rudder continuously to correct for long-term drift due to constant wind.

- Derivative (D): Corrects based on the rate of change of the error.

  - Example: Quickly adjust the rudder to counteract a sudden wave causing a rapid change in course.

By combining these three components, PID controllers provide a balanced approach to maintaining the desired setpoint in various control systems, offering both precision and stability.

2/2

4o

##### You said:

so in dont even undertstand steady state error .ecplain teh PID in relations to teh error it jelps eleimate but also epxlain that error

###### ChatGPT said:

ChatGPT

Let's dive deeper into understanding the types of errors PID controllers address, using simpler terms and relatable examples.

### Types of Errors:

- Current Error (Proportional Control):

  - Definition: The difference between the current state (e.g., temperature, position) and the desired state (setpoint).

  - Example: If your desired shower temperature is 40°C and the current temperature is 35°C, the error is 5°C.

- Accumulated Error (Integral Control):

  - Definition: The sum of past errors over time.

  - Example: If your shower has been 1°C too cold for the past 10 minutes, the accumulated error adds up those small discrepancies over time.

- Rate of Change of Error (Derivative Control):

  - Definition: The speed at which the error is changing.

  - Example: If the shower temperature starts dropping quickly, the rate of change of the error is high.

### Understanding Steady-State Error:

- Steady-State Error: This is the persistent difference between the desired state and the actual state after the system has settled. Even with proportional control, some systems might still have a small, consistent error.

  - Example: If you set your shower to 40°C but it stabilizes at 39°C, that 1°C difference is the steady-state error.

### How PID Helps Eliminate Errors:

- Proportional (P) Control:

  - Function: Corrects based on the current error.

  - Benefit: Provides immediate response to reduce the error.

  - Limitation: Might leave a steady-state error because it only reacts to the current error.

  - Example: If your car drifts to the left, you turn the wheel to the right proportionally to how far off you are.

- Integral (I) Control:

  - Function: Corrects based on the accumulation of past errors.

  - Benefit: Eliminates steady-state error by considering all past errors.

  - Limitation: Can cause the system to react slowly or overshoot if not tuned properly.

  - Example: If your car has been drifting to the left slightly for a long time, you gradually adjust the steering more to the right to correct for the accumulated drift.

- Derivative (D) Control:

  - Function: Corrects based on the rate of change of the error.

  - Benefit: Provides a damping effect, reducing overshoot and improving stability.

  - Limitation: Sensitive to noise and can overreact if not tuned properly.

  - Example: If your car suddenly starts drifting to the left quickly, you make a rapid adjustment to the right to counteract the drift.

### Putting It All Together:

Imagine you're riding a bicycle on a straight path:

- Proportional (P) Control:

  - You steer the bike based on how far you are from the center of the path.

  - If you're 5 inches off to the left, you turn the handlebars to the right proportionally.

- Integral (I) Control:

  - You also consider how long you've been off-center.

  - If you've been drifting to the left for a while, even if only slightly, you start turning the handlebars more to the right to compensate for the accumulated drift.

- Derivative (D) Control:

  - You react to how quickly you're drifting off the path.

  - If you start drifting quickly to the left, you make a quicker and more significant adjustment to the right to stop the drift.

### Mathematical Formula:

Control Output=Kp×Current Error+Ki×∑Past Errors+Kd×d(Error)dt\text{Control Output} = K_p \times \text{Current Error} + K_i \times \sum \text{Past Errors} + K_d \times \frac{d(\text{Error})}{dt}Control Output=Kp​×Current Error+Ki​×∑Past Errors+Kd​×dtd(Error)​

- KpK_pKp​: Proportional gain - adjusts based on current error.

- KiK_iKi​: Integral gain - adjusts based on accumulated error.

- KdK_dKd​: Derivative gain - adjusts based on the rate of change of error.

### Summary:

- P (Proportional): Reacts to the current error, providing an immediate correction.

  - Example: Turning the wheel based on how far off-center you are.

- I (Integral): Reacts to the accumulated error over time, eliminating steady-state error.

  - Example: Gradually adjusting the wheel more if you've been off-center for a while.

- D (Derivative): Reacts to the rate of change of the error, providing a predictive correction to prevent overshoot.

  - Example: Quickly adjusting the wheel if you start drifting quickly.

By understanding how each component of the PID controller addresses different aspects of the error, you can see how they work together to maintain the desired setpoint with precision and stability.

4o

##### You said:

use a rover movng to explain

###### ChatGPT said:

ChatGPT

Sure, let's use the example of a rover moving in a straight line to explain PID control and the types of errors it helps eliminate.

### Scenario: Rover Moving in a Straight Line

Imagine you have a rover that needs to move in a straight line. The goal is to keep the rover on a predefined path.

### Types of Errors:

- Current Error (Proportional Control):

  - Definition: The difference between the rover's current position and the desired position (path).

  - Example: If the rover is 10 cm to the left of the path, the error is 10 cm.

- Accumulated Error (Integral Control):

  - Definition: The sum of past errors over time.

  - Example: If the rover has been consistently 2 cm off the path for the past 5 seconds, these small errors add up.

- Rate of Change of Error (Derivative Control):

  - Definition: The speed at which the error is changing.

  - Example: If the rover suddenly starts drifting to the left at a rate of 3 cm per second, the rate of change of error is 3 cm/s.

### How PID Helps Eliminate Errors:

- Proportional (P) Control:

  - Function: Corrects based on the current error.

  - Benefit: Provides immediate response to reduce the error.

  - Limitation: Might leave a steady-state error because it only reacts to the current error.

  - Example: If the rover is 10 cm to the left of the path, it steers to the right proportionally to how far it is from the path. If it is 5 cm off, it steers less.

- Integral (I) Control:

  - Function: Corrects based on the accumulation of past errors.

  - Benefit: Eliminates steady-state error by considering all past errors.

  - Limitation: Can cause the system to react slowly or overshoot if not tuned properly.

  - Example: If the rover has been slightly off to the left for a long time, the integral action sums these small errors and makes a larger adjustment to bring it back to the path.

- Derivative (D) Control:

  - Function: Corrects based on the rate of change of the error.

  - Benefit: Provides a damping effect, reducing overshoot and improving stability.

  - Limitation: Sensitive to noise and can overreact if not tuned properly.

  - Example: If the rover suddenly starts drifting to the left quickly, the derivative action makes a rapid correction to the right to prevent it from drifting too far.

### Bringing It All Together:

#### Proportional (P) Control:

- Current Error: The rover's distance from the path.

- Correction: Steer the rover proportionally to the distance from the path.

  - Example: If the rover is 10 cm off, it makes a significant turn. If it is 2 cm off, it makes a smaller turn.

#### Integral (I) Control:

- Accumulated Error: The sum of all past deviations from the path.

- Correction: Adjust the rover's steering based on the total accumulated error.

  - Example: If the rover has been off the path for a long time, even by a small amount, the integral action increases the correction to bring it back to the path.

#### Derivative (D) Control:

- Rate of Change of Error: How quickly the rover is moving away from the path.

- Correction: Adjust the rover's steering based on how fast the error is changing.

  - Example: If the rover starts drifting away quickly, the derivative action applies a strong correction to counteract this drift.

### Real-World Example:

Imagine a rover moving on a straight line where there are some disturbances (like small obstacles or wind) causing it to drift off course:

- Proportional (P) Control:

  - The rover detects it is 10 cm off the path to the left. It steers to the right with a force proportional to 10 cm.

  - Result: The rover starts moving back towards the path but may not reach the exact center.

- Integral (I) Control:

  - The rover has been consistently 2 cm off the path to the left for the past few seconds. The integral action sums these small errors.

  - Result: The rover makes a stronger correction to the right to eliminate this accumulated error, bringing it back to the center over time.

- Derivative (D) Control:

  - The rover suddenly starts drifting to the left at a rate of 5 cm per second due to a gust of wind.

  - Result: The derivative action quickly adjusts the steering to the right to counteract this fast drift, preventing the rover from overshooting the path.

### Summary:

- P (Proportional): Corrects the current position error.

  - Example: Rover steers right if it is left of the path.

- I (Integral): Corrects the accumulated error over time.

  - Example: Rover makes a stronger correction if it has been off the path for a while.

- D (Derivative): Corrects the rate of change of the error.

  - Example: Rover makes a rapid adjustment if it starts drifting quickly.

By combining these three components, a PID controller ensures that the rover stays on its path by addressing the immediate error, accumulated errors over time, and the speed of error change, providing a smooth and accurate control.

4o

##### You said:

usea motor

###### ChatGPT said:

ChatGPT

Sure, let's use the example of controlling the speed of a motor to explain how PID control works and how it helps eliminate different types of errors.

### Scenario: Controlling the Speed of a Motor

Imagine you have a motor that you want to maintain at a constant speed, say 1000 RPM (revolutions per minute).

### Types of Errors:

- Current Error (Proportional Control):

  - Definition: The difference between the current speed and the desired speed (setpoint).

  - Example: If the motor is running at 950 RPM instead of 1000 RPM, the error is 50 RPM.

- Accumulated Error (Integral Control):

  - Definition: The sum of past errors over time.

  - Example: If the motor has been running at 990 RPM (10 RPM error) for the past 10 seconds, these small errors add up.

- Rate of Change of Error (Derivative Control):

  - Definition: The speed at which the error is changing.

  - Example: If the motor speed drops from 1000 RPM to 900 RPM rapidly, the rate of change of the error is high.

### How PID Helps Eliminate Errors:

- Proportional (P) Control:

  - Function: Corrects based on the current error.

  - Benefit: Provides immediate response to reduce the error.

  - Limitation: Might leave a steady-state error because it only reacts to the current error.

  - Example: If the motor is running at 950 RPM (50 RPM below the setpoint), proportional control increases the power to the motor proportionally to the error (50 RPM).

- Integral (I) Control:

  - Function: Corrects based on the accumulation of past errors.

  - Benefit: Eliminates steady-state error by considering all past errors.

  - Limitation: Can cause the system to react slowly or overshoot if not tuned properly.

  - Example: If the motor has been running consistently at 990 RPM (10 RPM below the setpoint), integral control sums these small errors and increases the power to eliminate the accumulated error over time.

- Derivative (D) Control:

  - Function: Corrects based on the rate of change of the error.

  - Benefit: Provides a damping effect, reducing overshoot and improving stability.

  - Limitation: Sensitive to noise and can overreact if not tuned properly.

  - Example: If the motor speed drops quickly from 1000 RPM to 900 RPM, derivative control increases the power quickly to counteract the rapid change in speed.

### Putting It All Together:

#### Proportional (P) Control:

- Current Error: The motor's speed difference from the setpoint.

- Correction: Increase or decrease the power to the motor proportionally to the speed error.

  - Example: If the motor is 50 RPM too slow, increase power proportionally to correct it.

#### Integral (I) Control:

- Accumulated Error: The sum of all past speed differences from the setpoint.

- Correction: Adjust the motor power based on the total accumulated error.

  - Example: If the motor has been slightly slow for a while, integral control increases power to correct the long-term error.

#### Derivative (D) Control:

- Rate of Change of Error: How quickly the motor speed is changing.

- Correction: Adjust the motor power to counteract rapid changes in speed.

  - Example: If the motor speed drops quickly, derivative control increases power quickly to stabilize it.

### Real-World Example:

Imagine you're controlling the speed of a motor where there are some disturbances (like changes in load) causing it to drift off the desired speed:

- Proportional (P) Control:

  - The motor speed is 950 RPM, which is 50 RPM below the desired 1000 RPM.

  - Proportional control increases the power to the motor to bring the speed up.

  - Result: The motor speed increases but might stabilize at, say, 990 RPM due to other influences, leaving a small steady-state error.

- Integral (I) Control:

  - The motor has been running at 990 RPM for some time, which is 10 RPM below the desired speed.

  - Integral control sums these small errors over time and gradually increases the power to eliminate the steady-state error.

  - Result: The motor speed eventually reaches and maintains 1000 RPM.

- Derivative (D) Control:

  - The motor speed suddenly drops from 1000 RPM to 900 RPM due to a sudden increase in load.

  - Derivative control reacts to this rapid change by quickly increasing the power to counteract the drop in speed.

  - Result: The motor speed stabilizes back to 1000 RPM quickly, preventing overshoot.

### Summary:

- P (Proportional): Corrects the current speed error.

  - Example: Increase power if the motor is running slower than desired.

- I (Integral): Corrects the accumulated speed error over time.

  - Example: Increase power gradually if the motor has been running slightly slower for a long period.

- D (Derivative): Corrects the rate of change of the speed error.

  - Example: Increase power quickly if the motor speed drops rapidly.

By combining these three components, a PID controller ensures that the motor maintains the desired speed by addressing the immediate speed error, accumulated errors over time, and the speed of error change, providing smooth and accurate control.

4o

##### You said:

ok lets runs a scenario and explain step by step how each of the pid would come in when a motor is running and its taret speed is 100rpm

###### ChatGPT said:

ChatGPT

Sure, let's walk through a scenario step-by-step to understand how each component of the PID controller (Proportional, Integral, Derivative) works when the target speed of a motor is 100 RPM.

### Scenario Setup

- Target Speed: 100 RPM

- Initial Speed: 80 RPM

- Disturbance: A sudden load change causing the speed to drop to 70 RPM

### Step-by-Step Explanation

- Initial Condition:

  - Current Speed: 80 RPM

  - Target Speed: 100 RPM

  - Error: 100 - 80 = 20 RPM (the motor is running 20 RPM too slow)

- Proportional (P) Control:

  - Proportional Action: The proportional controller multiplies the current error by a proportional gain KpK_pKp​.

  - Calculation: If Kp=1K_p = 1Kp​=1, the control output is 20×1=2020 \times 1 = 2020×1=20.

  - Effect: The motor controller increases the power to the motor to correct the 20 RPM error.

  - Result: The motor speed starts to increase but might not reach exactly 100 RPM due to the proportional-only control.

- Integral (I) Control:

  - Accumulated Error: Over time, the controller adds up all past errors to address any persistent offset.

  - Example: If the motor has been running at 95 RPM for a while, the accumulated error adds up (5 RPM error over time).

  - Calculation: If Ki=0.1K_i = 0.1Ki​=0.1 and the accumulated error over 10 seconds is 5×10=505 \times 10 = 505×10=50, the integral term is 0.1×50=50.1 \times 50 = 50.1×50=5.

  - Effect: The integral action increases the power to correct the long-term error.

  - Result: The motor speed adjusts to eliminate the steady-state error and stabilizes closer to 100 RPM.

- Derivative (D) Control:

  - Rate of Change of Error: The derivative controller responds to how quickly the error is changing.

  - Disturbance: A sudden load change causes the speed to drop from 100 RPM to 70 RPM quickly.

  - Calculation: If Kd=0.01K_d = 0.01Kd​=0.01 and the rate of change of error is (70−100)/1=−30(70 - 100) / 1 = -30(70−100)/1=−30 RPM per second, the derivative term is 0.01×(−30)=−0.30.01 \times (-30) = -0.30.01×(−30)=−0.3.

  - Effect: The derivative action provides a counteracting force to dampen the rapid change.

  - Result: The motor speed stabilizes more quickly, preventing overshoot and helping to maintain smooth control.

### Full PID Control:

When combined, the PID controller provides a comprehensive solution to maintain the target speed:

- Proportional (P):

  - Current Error: 20 RPM

  - Control Action: Adjusts power proportional to the error.

  - Effect: Quick correction to reduce the immediate error.

- Integral (I):

  - Accumulated Error: Sum of past errors.

  - Control Action: Adjusts power based on accumulated error.

  - Effect: Eliminates steady-state error over time.

- Derivative (D):

  - Rate of Change of Error: Speed of error change.

  - Control Action: Adjusts power based on how quickly the error is changing.

  - Effect: Dampens rapid changes, providing stability.

### Walkthrough with Values:

Let's assume:

- Kp=1K_p = 1Kp​=1

- Ki=0.1K_i = 0.1Ki​=0.1

- Kd=0.01K_d = 0.01Kd​=0.01

- Initial Speed at 80 RPM:

  - Error: 100 - 80 = 20 RPM

  - Proportional Output: 20×1=2020 \times 1 = 2020×1=20

  - Integral Output: Accumulated error = 0 initially.

  - Derivative Output: Initial change rate = 0.

- Speed increases to 90 RPM after initial correction:

  - Error: 100 - 90 = 10 RPM

  - Proportional Output: 10×1=1010 \times 1 = 1010×1=10

  - Integral Output: Accumulated error over time = 20+10=3020 + 10 = 3020+10=30, Integral action = 0.1×30=30.1 \times 30 = 30.1×30=3

  - Derivative Output: Rate of change (90−80)/1=10(90 - 80) / 1 = 10(90−80)/1=10 RPM/s, Derivative action = 0.01×10=0.10.01 \times 10 = 0.10.01×10=0.1

- Speed stabilizes at 100 RPM due to PID action:

  - Error: 0 RPM

  - Proportional Output: 0×1=00 \times 1 = 00×1=0

  - Integral Output: Accumulated error = Corrected over time.

  - Derivative Output: Rate of change stabilizes, action = Minimal.

- Sudden load change drops speed to 70 RPM:

  - Error: 100 - 70 = 30 RPM

  - Proportional Output: 30×1=3030 \times 1 = 3030×1=30

  - Integral Output: Accumulated error increases again.

  - Derivative Output: Rapid drop, rate = (70−100)/1=−30(70 - 100) / 1 = -30(70−100)/1=−30, Derivative action = 0.01×−30=−0.30.01 \times -30 = -0.30.01×−30=−0.3

  - Combined Output: PID controller quickly adjusts to counteract the drop and bring the speed back to 100 RPM.

### Summary:

- P (Proportional): Provides immediate correction based on current error.

- I (Integral): Eliminates steady-state error by considering past errors.

- D (Derivative): Prevents overshoot and improves stability by reacting to the rate of error change.

The PID controller ensures the motor maintains the desired speed (100 RPM) by continuously adjusting the power based on the current error, accumulated past errors, and the rate of change of error. This comprehensive approach provides smooth and accurate control.

Let's break down the scenario using a table to explain how each component of the PID controller (Proportional, Integral, Derivative) responds to disturbances and calculates adjustments to maintain a target motor speed of 100 RPM.

### Scenario Table with Calculations

| Time (s) | Target Speed (100 RPM) | Current Speed | Error (100 - Current Speed) | Proportional Output (P) | Accumulated Error (Integral, I) | Integral Output (I) | Rate of Change of Error (Derivative, D) | Derivative Output (D) | Total PID Output | Motor Response |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 100 | 80 | 100 - 80 = 20 | 20 * Kp = 20 | 0 | 0 | (80 - 80) / 1 = 0 | 0 | 20 | Increase power to correct initial error. |
| 1 | 100 | 90 | 100 - 90 = 10 | 10 * Kp = 10 | (20 + 10) = 30 | 30 * Ki = 3 | (90 - 80) / 1 = 10 | 10 * Kd = 0.1 | 13 | Adjust power based on accumulated and changing errors. |
| 2 | 100 | 95 | 100 - 95 = 5 | 5 * Kp = 5 | (30 + 5) = 35 | 35 * Ki = 3.5 | (95 - 90) / 1 = 5 | 5 * Kd = 0.05 | 8.55 | Further adjustment to reduce steady-state error. |
| 3 | 100 | 98 | 100 - 98 = 2 | 2 * Kp = 2 | (35 + 2) = 37 | 37 * Ki = 3.7 | (98 - 95) / 1 = 3 | 3 * Kd = 0.03 | 6.76 | Fine-tuning to approach target speed. |
| 4 | 100 | 100 | 100 - 100 = 0 | 0 * Kp = 0 | (37 + 0) = 37 | 37 * Ki = 3.7 | (100 - 98) / 1 = 2 | 2 * Kd = 0.02 | 3.74 | Speed is stabilized at target. |
| 5 | 100 | 70 | 100 - 70 = 30 | 30 * Kp = 30 | (37 + 30) = 67 | 67 * Ki = 6.7 | (70 - 100) / 1 = -30 | -30 * Kd = -0.3 | 6.74 | Responds to sudden drop, adjusts to correct error. |

- Explanation of Columns:

  - Time (s): Elapsed time in seconds.

  - Target Speed: Desired speed of the motor (100 RPM).

  - Current Speed: Actual speed of the motor at each time step.

  - Error: Difference between the target speed and current speed.

  - Proportional Output (P): Correction based on the current error multiplied by KpK_pKp​.

  - Accumulated Error (Integral, I): Sum of all past errors up to that point.

  - Integral Output (I): Correction based on the accumulated error multiplied by KiK_iKi​.

  - Rate of Change of Error (Derivative, D): Speed at which the error is changing (derivative of the error over time).

  - Derivative Output (D): Correction based on the rate of change of error multiplied by KdK_dKd​.

  - Total PID Output: Combined output of P, I, and D components.

  - Motor Response: Description of how the motor adjusts based on the total PID output.

### Summary:

- Proportional (P): Provides immediate correction proportional to the current error.

- Integral (I): Eliminates steady-state error by considering accumulated errors over time.

- Derivative (D): Damps oscillations and responds to rapid changes in error.

In the table above, each component of the PID controller works together to adjust the motor's power output dynamically, responding to changes in speed and disturbances to maintain the desired target speed of 100 RPM effectively.

3.5

##### You said:

add a clusm at teh ened of teh tbael thats shows teh current speed speed os te motrs in that row

###### ChatGPT said:

ChatGPT

Certainly! Let's add a column at the end of the table to show the current speed of the motor in each row, reflecting how the PID controller adjustments affect the motor's speed.

### Scenario Table with Calculations and Current Speed

| Time (s) | Target Speed (100 RPM) | Current Speed | Error (100 - Current Speed) | Proportional Output (P) | Accumulated Error (Integral, I) | Integral Output (I) | Rate of Change of Error (Derivative, D) | Derivative Output (D) | Total PID Output | Motor Response | Current Motor Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 100 | 80 | 100 - 80 = 20 | 20 * Kp = 20 | 0 | 0 | (80 - 80) / 1 = 0 | 0 | 20 | Increase power to correct initial error. | 82 |
| 1 | 100 | 90 | 100 - 90 = 10 | 10 * Kp = 10 | (20 + 10) = 30 | 30 * Ki = 3 | (90 - 80) / 1 = 10 | 10 * Kd = 0.1 | 13 | Adjust power based on accumulated and changing errors. | 93 |
| 2 | 100 | 95 | 100 - 95 = 5 | 5 * Kp = 5 | (30 + 5) = 35 | 35 * Ki = 3.5 | (95 - 90) / 1 = 5 | 5 * Kd = 0.05 | 8.55 | Further adjustment to reduce steady-state error. | 98 |
| 3 | 100 | 98 | 100 - 98 = 2 | 2 * Kp = 2 | (35 + 2) = 37 | 37 * Ki = 3.7 | (98 - 95) / 1 = 3 | 3 * Kd = 0.03 | 6.76 | Fine-tuning to approach target speed. | 99 |
| 4 | 100 | 100 | 100 - 100 = 0 | 0 * Kp = 0 | (37 + 0) = 37 | 37 * Ki = 3.7 | (100 - 98) / 1 = 2 | 2 * Kd = 0.02 | 3.74 | Speed is stabilized at target. | 100 |
| 5 | 100 | 70 | 100 - 70 = 30 | 30 * Kp = 30 | (37 + 30) = 67 | 67 * Ki = 6.7 | (70 - 100) / 1 = -30 | -30 * Kd = -0.3 | 6.74 | Responds to sudden drop, adjusts to correct error. | 97 |

- Explanation of Columns:

  - Time (s): Elapsed time in seconds.

  - Target Speed: Desired speed of the motor (100 RPM).

  - Current Speed: Actual speed of the motor at each time step.

  - Error: Difference between the target speed and current speed.

  - Proportional Output (P): Correction based on the current error multiplied by KpK_pKp​.

  - Accumulated Error (Integral, I): Sum of all past errors up to that point.

  - Integral Output (I): Correction based on the accumulated error multiplied by KiK_iKi​.

  - Rate of Change of Error (Derivative, D): Speed at which the error is changing (derivative of the error over time).

  - Derivative Output (D): Correction based on the rate of change of error multiplied by KdK_dKd​.

  - Total PID Output: Combined output of P, I, and D components.

  - Motor Response: Description of how the motor adjusts based on the total PID output.

  - Current Motor Speed: Updated speed of the motor after applying PID control adjustments.

### Summary:

- Proportional (P): Provides immediate correction proportional to the current error.

- Integral (I): Eliminates steady-state error by considering accumulated errors over time.

- Derivative (D): Damps oscillations and responds to rapid changes in error.

In this table, you can see how the PID controller components (P, I, D) work together to adjust the motor's speed dynamically in response to disturbances, gradually reducing error and stabilizing the motor speed at the target of 100 RPM. Each row reflects the cumulative effect of PID control over time, showing how the motor's speed adjusts step-by-step.

3.5

##### You said:

show hwo u come up with it

i meant in teh tbale

###### ChatGPT said:

ChatGPT

Certainly! Let's go through each row of the table step by step, showing how the calculations for Proportional (P), Integral (I), Derivative (D), and the resulting Motor Response are derived.

### Scenario Table with Calculations and Current Speed

| Time (s) | Target Speed (100 RPM) | Current Speed | Error (100 - Current Speed) | Proportional Output (P) | Accumulated Error (Integral, I) | Integral Output (I) | Rate of Change of Error (Derivative, D) | Derivative Output (D) | Total PID Output | Motor Response | Current Motor Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 100 | 80 | 100 - 80 = 20 | 20 * Kp = 20 | 0 | 0 | (80 - 80) / 1 = 0 | 0 | 20 | Increase power to correct initial error. | 82 |
| 1 | 100 | 90 | 100 - 90 = 10 | 10 * Kp = 10 | (20 + 10) = 30 | 30 * Ki = 3 | (90 - 80) / 1 = 10 | 10 * Kd = 0.1 | 13 | Adjust power based on accumulated and changing errors. | 93 |
| 2 | 100 | 95 | 100 - 95 = 5 | 5 * Kp = 5 | (30 + 5) = 35 | 35 * Ki = 3.5 | (95 - 90) / 1 = 5 | 5 * Kd = 0.05 | 8.55 | Further adjustment to reduce steady-state error. | 98 |
| 3 | 100 | 98 | 100 - 98 = 2 | 2 * Kp = 2 | (35 + 2) = 37 | 37 * Ki = 3.7 | (98 - 95) / 1 = 3 | 3 * Kd = 0.03 | 6.76 | Fine-tuning to approach target speed. | 99 |
| 4 | 100 | 100 | 100 - 100 = 0 | 0 * Kp = 0 | (37 + 0) = 37 | 37 * Ki = 3.7 | (100 - 98) / 1 = 2 | 2 * Kd = 0.02 | 3.74 | Speed is stabilized at target. | 100 |
| 5 | 100 | 70 | 100 - 70 = 30 | 30 * Kp = 30 | (37 + 30) = 67 | 67 * Ki = 6.7 | (70 - 100) / 1 = -30 | -30 * Kd = -0.3 | 6.74 | Responds to sudden drop, adjusts to correct error. | 97 |

### Explanation of Calculations:

- Row 0 (Time = 0 seconds):

  - Current Speed: 80 RPM

  - Error: 100 - 80 = 20 RPM

  - Proportional Output (P): 20×Kp20 \times Kp20×Kp

  - Integral Output (I): Sum of errors so far multiplied by KiKiKi

  - Derivative Output (D): Rate of change of error multiplied by KdKdKd

  - Total PID Output: Sum of P, I, and D outputs

  - Motor Response: Increase power to correct initial error

  - Current Motor Speed: 82 RPM (After adjustment)

- Row 1 (Time = 1 second):

  - Current Speed: 90 RPM

  - Error: 100 - 90 = 10 RPM

  - Proportional Output (P): 10×Kp10 \times Kp10×Kp

  - Integral Output (I): Updated sum of errors multiplied by KiKiKi

  - Derivative Output (D): Rate of change of error multiplied by KdKdKd

  - Total PID Output: Sum of P, I, and D outputs

  - Motor Response: Adjust power based on accumulated and changing errors

  - Current Motor Speed: 93 RPM (After adjustment)

- Row 2 (Time = 2 seconds):

  - Current Speed: 95 RPM

  - Error: 100 - 95 = 5 RPM

  - Proportional Output (P): 5×Kp5 \times Kp5×Kp

  - Integral Output (I): Updated sum of errors multiplied by KiKiKi

  - Derivative Output (D): Rate of change of error multiplied by KdKdKd

  - Total PID Output: Sum of P, I, and D outputs

  - Motor Response: Further adjustment to reduce steady-state error

  - Current Motor Speed: 98 RPM (After adjustment)

- Row 3 (Time = 3 seconds):

  - Current Speed: 98 RPM

  - Error: 100 - 98 = 2 RPM

  - Proportional Output (P): 2×Kp2 \times Kp2×Kp

  - Integral Output (I): Updated sum of errors multiplied by KiKiKi

  - Derivative Output (D): Rate of change of error multiplied by KdKdKd

  - Total PID Output: Sum of P, I, and D outputs

  - Motor Response: Fine-tuning to approach target speed

  - Current Motor Speed: 99 RPM (After adjustment)

- Row 4 (Time = 4 seconds):

  - Current Speed: 100 RPM

  - Error: 100 - 100 = 0 RPM

  - Proportional Output (P): 0×Kp0 \times Kp0×Kp

  - Integral Output (I): Updated sum of errors multiplied by KiKiKi

  - Derivative Output (D): Rate of change of error multiplied by KdKdKd

  - Total PID Output: Sum of P, I, and D outputs

  - Motor Response: Speed is stabilized at target

  - Current Motor Speed: 100 RPM (Stabilized)

- Row 5 (Time = 5 seconds):

  - Current Speed: 70 RPM

  - Error: 100 - 70 = 30 RPM

  - Proportional Output (P): 30×Kp30 \times Kp30×Kp

  - Integral Output (I): Updated sum of errors multiplied by KiKiKi

  - Derivative Output (D): Rate of change of error multiplied by KdKdKd

  - Total PID Output: Sum of P, I, and D outputs

  - Motor Response: Responds to sudden drop, adjusts to correct error

  - Current Motor Speed: 97 RPM (After adjustment)

### Summary:

In each row of the table:

- Proportional (P): Computes an output based on the current error.

- Integral (I): Accumulates past errors to eliminate steady-state error.

- Derivative (D): Reacts to the rate of change of error to improve response.

- Total PID Output: Combines P, I, and D outputs to determine the motor's adjustment.

- Motor Response: Describes how the motor adjusts its speed based on the calculated PID output.

- Current Motor Speed: Reflects the motor's speed after applying PID control adjustments.

This table and explanation demonstrate how the PID controller dynamically adjusts the motor's speed to maintain the target of 100 RPM, responding to errors and disturbances over time.

3.5

##### You said:

how is totla pid output elated to current mototr speed

###### ChatGPT said:

ChatGPT

The total PID output is directly related to the current motor speed because it determines how much the motor's speed should be adjusted to reach and maintain the target speed (in this case, 100 RPM). Here’s how the total PID output affects the current motor speed:

- Proportional (P) Component: This component provides an immediate correction proportional to the current error (difference between the target speed and current speed). If the error is large, the proportional output will be larger, signaling the motor to adjust more aggressively.

- Integral (I) Component: The integral component considers accumulated past errors. It helps eliminate steady-state error by continuously summing up past errors multiplied by a constant KiK_iKi​. This means if the motor has consistently been below or above the target speed over time, the integral term will gradually increase or decrease the motor speed to bring it closer to the target.

- Derivative (D) Component: The derivative component responds to the rate of change of the error. It helps dampen oscillations and reduces overshoot by considering how quickly the error is changing over time. If the error is decreasing rapidly (indicating the motor is approaching the target quickly), the derivative term will reduce the correction to prevent overshooting.

- Total PID Output: This is the sum of the proportional, integral, and derivative outputs:

Total PID Output=P+I+D\text{Total PID Output} = P + I + DTotal PID Output=P+I+D

The total PID output directly determines how much the motor's speed should change in response to the current error and its historical behavior (accumulated errors and rate of change of errors).

- Motor Response: Based on the total PID output, the motor adjusts its speed. A positive PID output increases the motor speed, while a negative output decreases it.

- Current Motor Speed: After applying the total PID output, the motor's speed is adjusted accordingly. For example, if the total PID output suggests an increase in speed (positive output), the current motor speed will increase. Conversely, if the output suggests a decrease (negative output), the current motor speed will decrease.

### Example Application:

- Scenario: At a certain time, the motor's target speed is 100 RPM, but the current speed is 80 RPM (error of 20 RPM).

- Calculations:

  - Proportional Output (P): 20×Kp20 \times Kp20×Kp

  - Integral Output (I): Accumulated errors over time multiplied by KiKiKi

  - Derivative Output (D): Rate of change of error multiplied by KdKdKd

  - Total PID Output: Sum of P, I, and D

  - Motor Response: Adjust motor speed based on the total PID output

  - Current Motor Speed: Reflects the motor's speed after adjustment based on the total PID output

In summary, the total PID output is a mathematical sum of the proportional, integral, and derivative components, each of which contributes to adjusting the motor's speed to maintain the desired target speed over time, considering both immediate and historical error information.